TUTORIAL 1: Numbering Systems

- Convert the following to decimal numbers.

- 111001112

Answer

111001112 = (1x27)+(1 x26)+(1 x25)+(0 x24)+(0 x23)+(1 x22)+(1 x21)+(1 x20) = 128 + 64 + 32 + 0 + 0 + 4 + 2 + 1 = 23110

- 55778

Answer

55778 = (5 x83)+(5 x82)+(7 x81)+(7 x80) = 2560 + 320 +56 + 7 = 294310

- ABCD H

Answer

ABCD H = (A x163)+(B x162)+(C x161)+(D x160) = 40960 + 2816 + 192 + 13 = 4398110

- Convert the followings to Base-2, Base-8 and Base-16 respectively.

- 3064 D

| Answer |  |  |
| --- | --- | --- |
| Base-2 | Base-8 | Base-16 |
| 2) 3064 - 0 2) 1532 – 0 2) 766 – 0 2) 383 – 1 2) 191 – 1 2) 95 – 1 2) 47 – 1 2) 23 – 1 2) 11 – 1 2) 5 – 1 2) 2 – 0 1 Answer: 1011 1111 10002 | 8) 3064 – 0 8) 383 – 7 8) 47 – 7 5 Answer: 57708 | 16) 3064 – 8 16) 191 – F B Answer: BF816 |

- 201610

| Answer |  |  |
| --- | --- | --- |
| Base-2 | Base-8 | Base-16 |
| 2) 2016 - 0 2) 1008 – 0 2) 504 – 0 2) 252 – 0 2) 126 – 0 2) 63 – 1 2) 31 – 1 2) 15 – 1 2) 7 – 1 2) 3 – 1 1 Answer: 111 1110 00002 | 8) 2016 – 0 8) 252 – 4 8) 31 – 7 3 Answer: 37408 | 16) 2016– 0 16) 126 – E 7 Answer: 7E016 |

- 2899 base-10

| Answer |  |  |
| --- | --- | --- |
| Base-2 | Base-8 | Base-16 |
| 2) 2899 – 1 2) 1449 – 1 2) 724 – 0 2) 362 – 0 2) 181 – 1 2) 90 – 0 2) 45 – 1 2) 22 – 0 2) 11 – 1 2) 5 – 1 2) 2 – 0 1 Answer: 1011 0101 00112 | 8) 2899 – 3 8) 362 – 2 8) 45 – 5 5 Answer: 55238 | 16) 2899 – 3 16) 181 – 5 B Answer: B5316 |

- Convert the following numbers to the respective base.

- 11111010001111002 to base-8 and base-16 respectively

| Answer |  |
| --- | --- |
| Base-8 | Base-16 |
| 11111010001111002 = 1/111/101/000/111/100 = 1 7 5 0 7 48 | 11111010001111002 = 1111/1010/0011/1100 = F A 3 C16 |

- 11000011101001012 to base-8 and base-16 respectively

| Answer |  |
| --- | --- |
| Base-8 | Base-16 |
| 11000011101001012 = 1/100/001/110/100/101 = 1 4 1 6 4 58 | 11000011101001012 = 1100/0011/1010/0101 = C 3 A 516 |

- 1EFH to base-2 and base-8 respectively

| Answer |  |
| --- | --- |
| Base-2 | Base-8 |
| 1EFH = 0001 1110 11112 | 1EFH = 0001 1110 11112 = 000/111/101/111 = 0 7 5 7 8 |

- 257ACH to base-2 and base-8 respectively

| Answer |  |
| --- | --- |
| Base-2 | Base-8 |
| 257ACH = 0010 0101 0111 1010 11002 | 257ACH = 0010 0101 0111 1010 11002 = 00/100/101/011/110/101/100 = 0 4 5 3 6 5 4 8 |

- 7548 to base-16

Answer

7548 = 111 101 1002 = 1/1110/1100 = 1 E C16

- 44716 to base-8

Answer

44716 = 0100 0100 01112 = 010/001/000/111 = 2 1 0 78

- Perform the following conversions. You are required to show the working steps clearly. If the operation(s) is illogical, explain the reason.

(a)	165810 to hexadecimal number	(PYP-08/14: 2 marks)

Answer

16) 1658 – A 16) 103 – 7 6 Answer = 67A16

(b)	7658 to hexadecimal number	(PYP-08/14: 2 marks)

Answer

7658 = 111 110 1012 = 1/1111/0101 = 1 F 516

(c)	673247 to decimal number	(PYP-08/13: 2 marks)

Answer

673247 = Illogical. Because base-7 does not include 7.

(d)	8739010 to base-5 number	(PYP-08/13: 2 marks)

Answer

5)87390 – 0 5) 17478 – 3 5) 3495 – 0 5) 699 – 4 5) 139 – 4 5) 27 – 2 5) 5 – 0 1 Answer: 102440305

(e)	392088 to decimal number	(PYP-04/13: 2 marks)

Answer

392088 = Illogical. Because base-8 consists of numbers range from 0 to 7 only.

- Perform the following conversions. You are required to show the working steps clearly. If the operation(s) is illogical, explain the reason.

(a)	123010 to base-6 number	(PYP-04/13: 2 marks)

Answer

6) 1230 – 0 6) 205 – 1 6) 34 – 4 5 Answer: 54106

(b)	152810 to hexadecimal number	(PYP-08/15: 2 marks)

Answer

16) 1528 – 8 16) 95 – F 5 Answer: 5F816

(c)	62178 to decimal number	(PYP-08/15: 2 marks)

Answer

62718 = (6 x83)+(2 x82)+(1 x81)+(7 x80) = 3072 + 128 + 8 + 7 = 321510

(d)	3728 to hexadecimal number	(PYP-08/15: 2 marks)

Answer

3728 = 011 111 0102 = 0/1111/1010 = 0 F A16

(e)	23010 to base-7 number	(PYP-01/14: 2 marks)

Answer

7) 230 – 6 7) 32 – 4 4 Answer : 4467

- Perform the following operations and show the answers in the respective number base. You are required to show your working steps clearly

[NOTE: every 16, carry 1 to next]

- ABC16 + FFF16	(PYP-01/14: 3 marks)

Answer

A B C H + F F F H 1 A B B16

- 1258 – 778	(PYP-01/14: 2 marks)

Answer

1 2 58 - 7 78 2 68

- 10112 x 1012

Answer

1 0 1 12 X 1 0 12 1 0 1 1 1 0 1 1 0 1 1 0 1 1 12

- 110112 x 10112

Answer

1 1 0 1 12 X 1 0 1 12 1 1 0 1 1 1 1 0 1 1 1 1 0 1 1 0 1 0 0 1 0 1 0 0 12

- 100111002 + 010010112

Answer

1 0 0 1 1 1 0 02 + 0 1 0 0 1 0 1 12 1 1 1 0 0 1 1 12

- Perform the following operations and show the answers in the respective number base. You are required to show your working steps clearly

- 4618 + 5158

Answer

4 6 18 + 5 1 58 1 1 7 68

- 2247 - 1367

Answer

2 2 47 - 1 3 67 557

- 173H x AAH

Answer

1 7 3 H X A A H E 7 E E 7 E F 6 5 E H

- 111110102 + 11108 (Show your final answer in Hex format.)	(PYP-04/13: 3 marks)

Answer

111110102 = 1111/1010 = F A16 11108 = 001 001 001 0002 = 0010/0100/1000 = 24816 111110102 FA16 + 11108 + 24816 34216

- 306416 x 2138 (Show your final answer in Hex format.)	(PYP-08/12: 3 marks)

Answer

2138 = 010 001 0112 =0/1000/1011 = 0 8 B16 3 0 6 416 X 8 B16 2 1 4 4 C 1 8 3 2 0 1 A 4 6 4 C16

TUTORIAL 2: Numerical Data Representation

Section A: Signed Number

- Under what circumstances the Two’s Complement is used?

Answer

Two’s complement method can be used to: - - Represent negative value - Perform subtraction

- Convert the 8-bit binary number 11010111 into decimal number if the binary number is a(n):

- Unsigned number	(PYP-04/14: 1 mark)

Answer

110101112 = (1x27)+(1 x26)+(0 x25)+(1 x24)+(0 x23)+(1 x22)+(1 x21)+(1 x20) = 128+64+0+16+0+4+2+1 = 21510

- Signed number	(PYP-04/14: 2 marks)

Answer

110101112 = (1x27)+(1 x26)+(0 x25)+(1 x24)+(0 x23)+(1 x22)+(1 x21)+(1 x20) = -128+64+0+16+0+4+2+1 = -4110

- Differentiate between carry flag and overflow flag. Complete the following table.

| Answer |  |  |
| --- | --- | --- |
| Flag | Carry | Overflow |
| Definition | Occurs when the result of an arithmetic operation exceeds the fixed number of bits allocated, without regard to sign | Occurs when the result of an arithmetic operation does not fit into the fixed numbers of bits available for the result |
| Detect in signed or unsigned number? | Unsigned numbers | Signed numbers |
| How to detect? | Extra “1” bit generated | - Both operands have the same sign - The result has opposite sign |
| Example | (-4) + (-2) 1100 No overflow +1110 Carry 11010 The result is correct | (+4) + (+6) 0100 Overflow +0110 No carry 1010 The result is incorrect |

- Assuming an 8-bit system is used, show how the following operation is solved through Two’s Complement method.

- 12410 - 610

Verify and comment the answer.					            (PYP-04/14: 5 marks)

Answer

12410 = 0111 11002 One’s complement = 1000 0011 + 1 Two’s complement = 1000 01002 (-12410) 610 = 0000 01102 One’s complement = 1111 1001 + 1 Two’s complement = 1111 10102 (-610) -12410-610 = (-12410) + (-610) 1000 01002 -12410 + 1111 10102 + - 610 (1)0111 11102 - 13010 0111 11102 = +12610 This is invalid. Because both binary (0111 11102 = +12610) and decimal (- 13010) computation return different results.

- Assuming that an 8-bit system is being applied, perform the binary subtraction operation for the following decimal numbers using Two’s Complement method.

65 - 54

Verify your answer by showing the answer in signed decimal value.	            (PYP-08/13: 5 marks)

Answer

65 10 = 0100 00012 5410 = 0011 01102 One’s complement = 1100 1001 + 1 Two’s complement = 1100 10102 (-5410) 6510-5410 = (6510) + (-5410) 0100 00012 6510 + 1100 10102 + - 5410 (1)0000 10112 1110 0000 10112 = 8+2+1 = +1110

- Assuming an 8-bit system is used (i.e. the system uses 8 bits to represent an integer). Given the following decimal numbers:

-12 + -8

- Solve the above operation using two’s complement method.	            (PYP-08/15: 5 marks)

Answer

To prove 1110 110 -20 20 = 0001 0100 1’s complement = 1110 1011 + 1 2’s complement = 1110 1100 So 1110 1100 = -20 Or refer to (b) 1210 = 0000 11002 One’s complement = 1111 0011 + 1 Two’s complement = 1111 01002 (-1210) 810 = 0000 10002 One’s complement = 1111 0111 + 1 Two’s complement = 1111 10002 (-810) -1210-810 = (-1210) + (-810) 1111 01002 - 1210 + 1111 10002 + - 810 (1)1110 11002 - 2010

- Verify your answer by showing the answer in signed decimal value.       (PYP-08/13: 3 marks)

Answer

1110 11002 = -128+64+32+8+4 = -2010

- Justify the validity of the answer obtained.			            (PYP-08/13: 1 mark)

Answer

This is valid. Because both binary and decimal computation return the same result.

- Does overflow occur? Justify your answer.		           	            (PYP-08/13: 1 mark)

Answer

Overflow does not occur. Because the result having the same sign as both operands.

- Assuming an 8-bit system is involve.

- Solve the following operation using Two’s Complement method:	            (PYP-08/14: 5 marks)

(-910) + (-810)

Answer

910 = 0000 10012 One’s complement = 1111 0110 + 1 Two’s complement = 1111 01112 (-910) 810 = 0000 10002 One’s complement = 1111 0111 + 1 Two’s complement = 1111 10002 (-810) 1111 01112 - 910 + 1111 10002 + - 810 (1)1110 11112 - 1710

- Verify your answer by showing the answer in signed decimal value.        (PYP-08/14: 1 mark)

Answer

1110 11112 = -128+64+32+8+4+2+1 = - 1710

- Justify the validity of the answer obtained.			            (PYP-08/14: 4 marks)

Answer

This is valid. Because both binary and decimal computation return the same result.

- Does overflow or/and carry occur?

Answer

Overflow = No Carry = Yes

Section B: Floating Point Number

- Perform the following number conversions. Show your conversion steps clearly. If the operation is illogical, explain the reason.

- 30.3010 to Binary 	(PYP-08/14: 2 marks)

Answer

3010 = 111102 0.30 x 2 = 0.60 0.60 x 2 = 1.20 0.20 x 2 = 0.40 0.4 x 2 = 0.8 0.8 x 2 = 1.6 0.6 x 2 30.3010 = 11110.010012

- 123.1235 to Decimal	(PYP-08/14: 2 marks)

Answer

123.1235 = (1 x 52)+(2 x 51)+(3 x 50)+ (1 x 5-1)+(2 x 5-2)+(3 x 5-3) = (25+10+3)+(0.2+0.08+0.024) = 38.30410

- 100100011111.112 to Octal	(PYP-08/14: 2 marks)

Answer

100100011111.112 = 100/100/011/111.110 = 4 4 3 7 . 68

- Perform the following operations. Show your working steps clearly. If the operation is illogical, explain the reason.

- Convert 6A.9610 to hexadecimal number

Answer

Illogical. Because base-10 does not include A.

- Convert 1807.6510 into a hexadecimal number	(PYP-08/13: 2 marks)

Answer

16) 1807 -F 16) 112 –0 7 0.65 x 16= 10.4 .4 x 16= 6.4 1807.6510 = 70F.A616

- Convert 101011.01112 into a decimal number	(PYP-04/13: 2 marks)

Answer

101011.01112 = (1 x 25) + (0 x 24) + (1 x 23) + (0 x 22) + (1 x 21) + (1 x 20).(0 x 2-1) + (1 x 2-2) + (1 x 2-3) + (1 x 2-4) = (32+0+8+0+2+1)+(0+0.25+0.125+0.0625) = 43.437510

- 111100110011.110000012 + 20.510. Show your answer in Hex format.	(PYP-08/12: 3 marks)

Answer

111100110011.110000012 = 1111/0011/0011.1100/0001 = F33.C116 20 = 101002 0.5 x 2 =1.0 20.510 = 10100.12 = 1/0100.1000 = 14.816 F 3 3 . C 1H + 1 4 . 8 0 H F 4 8 . 4 1 H

- Given that:

- An Excess-52 notation is applied.

- The implied decimal point is at the beginning of the mantissa.

- A “5” is used to represent a positive number and a “9” is used to represent a negative number.

- Convert -357.24610 to the SEEMMMMM format.  	                         (PYP-04/14: 2 marks)

Answer

-357.24610 = - 0.35724(6) x 103 = 9 55 35725 (SEEMMMMM format)

- Convert 55220311 to scientific notation.

Answer

55220311 = +0.20311 x 100 (scientific notation)

- Convert 95575321 to scientific notation.

Answer

95575321 = - 0.75321 x 103 (scientific notation)

- Convert 30.81510 to the SEEMMMMM format.

Answer

30.81510 = + 0.30815 x 102 = 5 54 30815 (SEEMMMMM format)

- The following decimal numbers are stored in excess-50 floating point format. A “1”is used to represent a negative sign, and a “5” for positive sign.

- Add these two numbers. Present your result in standard decimal sign-and-magnitude notation.

55020311

15375321

(PYP-08/13: 5 marks)

Answer

- Adjust exponent: 5 53 00020311 - Addition : 1 53 75321 + - SEEMMMMM : 1 53 75300689 - Sign-Magnitude : - 0.75301 x 103

- Multiply these two numbers. Present your result in standard decimal sign-and-magnitude notation.

15176323

15485496					                        (PYP-08/13: 5 marks)

Answer

- Adjust exponent: 51 + 54 – 50 = 55 = 105 - Multiply : - 0.76323 x - 0.85496 = 0.65253(11208) - SEEMMMMM : 5 55 65253 - Sign-magnitude : 0.65253 x 105

- Find the difference of these two numbers. Present your result in standard decimal sign-and-magnitude notation.

55152295

15256608		x – (-y) = x+y		                        (PYP-04/14: 5 marks)

Answer

- Adjust exponent : 5 52 052295 - Addition : 1 52 566080 - - SEEMMMMM : 5 52 61838 - Sign-Magnitude : +0.61838 x 102

- The floating point decimal numbers below are stored in the form of SEEMMMMM where the exponent is stored in excess-50 with the implied decimal point at the beginning of the mantissa. A 4 in the sign position indicates a positive number and a 3 indicates a negative number:

45320460

35520112

- Add these two numbers. Show the result in sign-magnitude notation.      (PYP-08/12: 3 marks)

Answer

- Adjust exponent : 4 55 0020460 - Addition : 3 55 20112 + - SEEMMMMM : 3 55 19907(40) - Sign-Magnitude : -0.19907 x 105

- Multiply these two numbers. Show the result in sign-magnitude notation. (PYP-08/12: 3 marks)

Answer

- Adjust exponent: 53 + 55 – 50 = 58 = 108 - Multiply : 0.20460 x - 0.20112 = - 0.041149(152) = - 0.41149 x 10-1 - Sign-magnitude : - 0.41149 x 10-1 x108 = - 0.41149 x 107 - SEEMMMMM : 3 57 41149

Extra Example on Divide:

05220000

04712500

1) adjust exponent	: 52-47+50 = 55

2) divide 		: 0.22/.125

: 1.76

: 0.176 x 101

3) sign-magnitude	: .176 x 101 x 105

: .176 x 106

4) SEEMMMMM	: 05617600

- Show how the number -5.510 is stored in the computer’s storage using IEEE754 32-bit single precision format. You are required to show your conversion steps clearly.     (PYP-01/14: 6 marks)

Answer

-5.510 = -101.12 - Adjust exponent : -1.011 x 22 - Exponent (D) : 127 + 2 = 129 - Exponent (B) : 1000 0001 - Sign : 1 - IEEE754 (S.P) : 1 1000 0001 0112..

- Represent the binary number -10111.012 into IEEE754 single precision format. You are required to show your conversion steps clearly.	(PYP-08/15: 5 marks)

Answer

-10111.012 - Adjust exponent : -1.011101 x 24 - Exponent (D) : 127 + 4 = 131 - Exponent (B) : 1000 0011 - Sign : 1 - IEEE754 (S.P) : 1 1000 0011 0111012..

- Given a decimal number “-30.8125”, how this notation can be represented in the IEEE754 single precision notation. You are required to show your working steps.	(PYP-08/12: 5 marks)

Answer

-30.812510 = -11110.11012 - Adjust exponent : -1.11101101 x 24 - Exponent (D) : 127 + 4 = 131 - Exponent (B) : 1000 0011 - Sign : 1 - IEEE754 (S.P) : 1 1000 0011 111011012..

- Given an IEEE754 single precision notation below, show how this notation can be represented in a sign-magnitude notation. You are required to show your working steps.

1 1000 0010 0100 1000 0000 0000 0000 000

Assuming that excess-127 is applied.				          (PYP-08/11: 5 marks)

Answer

1 1000 0010 0100 1000 0000 0000 0000 000 - IEEE754 (S.P) : 1 1000 0010 0100 1000 0000 0000 0000 000 - Sign : 1 = negative - Exponent (B) : 1000 0010 - Exponent (D) : 130 = 23 - Binary number : -1.01001 x 23 = -1010.012 - Decimal number: -10.2510 - Sign-magnitude: - 0.1025 x 102

TUTORIAL 3: Addressing Data in Memory and Segment

- Explain each of the following terms:

- Segment

Answer

Segment - Segments are defined as special areas of memory containing the code, data and stack information. - They are used to keep track of locations of individual program segments. - They are categorized into 3 key types: Code segment, data segment and stack segment. - Each segment can be up to 64KB in size.

- Offset

Answer

Offset - It is defined as the distance in bytes from the segment address to another location within the segment. - It uses 16-bit addressing range from 0000H to FFFFH

- Register

Answer

Register - It is defined as high speed storage location inside the CPU. - It is used to store information temporarily.

- Explain the purpose of the following items:

- Code segment 							         (PYP-01/16: 1 mark)

Answer

- Hold the machine instructions.

- Code segment (CS) register					         (PYP-01/16: 1 mark)

Answer

- Contains starting address of a program’s code segment.

- Instruction pointer (IP) register					         (PYP-01/16: 1 mark)

Answer

- Contains the offset address of the next instruction that is to execute. - Indicates the current instruction within the currently executing code segment.

- Accumulator (AX)

Answer

It is used for operations involving I/O and most arithmetic.

- Count register (CX)

Answer

It contains a value to control the number for times a loop is repeated or used for computation.

- Parity flag (PF)

Answer

It is used for error checking.

- (a)	List and explain FOUR (4) main segment in a CPU.		         (PYP-04/14: 4 marks)

Answer

- Code segment - Data segment - Extra segment - Stack segment

(b)	Explain a similarity and THREE (3) differences between a register and a Random Access Memory (RAM).						        (PYP-08/13: 5 marks)

| Answer |  |  |
| --- | --- | --- |
|  | Register | Memory |
| Similarity | Provide storage |  |
| Differences: (1) Data stored | Normal-byte sequence | Reversed-byte sequence |
| (2) Referred by | Name | Address |
| (3) Location | Inside CPU | Outside CPU |

(c) Differentiate between a segment:offset address and an absolute address.

| Answer |  |  |
| --- | --- | --- |
|  | Segment:Offset address | Absolute address |
| Characteristic | Logical address | Physical address |
| Size | 16-bit : 16-bit | 20-bit |
| Aim | To provide real address | To directly reference a specific location in memory |

- Perform the following calculations and show your answers in hexadecimal format. Draw a diagram for each of them to indicate how the respective answer would store in the AX register. You are required to show your working steps clearly.

- 738 + 258							        (PYP-08/11: 4 marks)

Answer

738 + 258 = 1208 = 5016 AH: 00H AL: 50H AX register

- 11112 x 1112 x 112						            (PYP-08/11: 4 marks)

Answer

1 1 1 1 X 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 0 1 0 0 1 X 1 1 1 1 0 1 0 0 1 1 1 0 1 0 0 1 1 0 0 1 1 1 0 1 12 = 13BH AH: 01H AL: 3BH AX register:

- Given the information in the table below.

| Code segment (CS) register: 02B3H Data segment (DS) register: 26D2H Stack segment (SS) register: 09AFH | Base Pointer (BP) register: 2062H Stack Pointer (SP) register: 0094H Instruction Pointer (IP) register: 0025H |
| --- | --- |

- Calculate the absolute address for the next instruction to be executed by the CPU.

Answer

Absolute address = (Segment address x 10H) + Offset Address = (02B3H x 10H) + 0025H = 02B55H

- Calculate the corresponding 20-bit absolute memory address using the SS:SP.

(PYP-08/13: 4 marks)

Answer

Absolute address = (Segment address x 10H) + Offset Address = (09AFH x 10H) + 0094H = 09B84H

- Given a 16-bit CS register consists of the hexadecimal value 12AB and the 16-bit IP register consists of the hexadecimal value 0020. Find the absolute address of the instruction and show the answer in 20-bit binary format.					      (PYP-08/14: 4 marks)

Answer

Absolute address = (Segment address x 10H) + Offset Address = (12ABH x 10H) + 0020H = 12AD0H = 0001 0010 1010 1101 0000

- Complete the following table.

| Answer |  |  |  |
| --- | --- | --- | --- |
| Register | Definition | A high speed storage located inside CPU |  |
|  | Characteristics | Used to store information temporarily |  |
| Types of register | General purpose | Address | Status |
| Aim | Used for arithmetic & data movement operation | Used for indexed addressing | Used to indicate the status of various activities |
| Size | 8-bit / 16-bit | 16-bit | 1-bit |
| Examples | 16-bit: AX,BX,CX,DX 8-bit: AH, AL, BH, BL, CH, CL, DH, DL | Segment: CS, DS, ES, SS Pointer: IP, BP, SP Index: SI, DI | OF DF IF TF SF ZF AF PF CF |

- Determine which register is/are used for the following purposes:-

- When the result of an arithmetic or logical operation generates a result of zero.

Answer

ZF

- Used for error checking when there is a possibility that data might be altered or corrupted.

Answer

PF

- When the result of an unsigned arithmetic operation is too large to fit into the destination.

Answer

CF

(d) 	Used for arithmetic and data movement.

Answer

AX

(e) 	Used for counting loops.

Answer

CX

- Write down the values of the Carry, Sign, Zero and Overflow flags after each instruction has executed. Show your working steps clearly.

MOV  AX, 6120H

ADD  AL,  AAH

ADD  AH,  FFH

ADD  AX,  2

| Answer |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Instructions | Operations | CF | SF | ZF | OF |
| MOV AX, 6120H | AX = 0110 0001 0010 00002 | NC | PL | NZ | NV |
| ADD AL, AAH | AL = 0010 00002 AAH = 1010 10102 + AL = 1100 10102 | NC | NG | NZ | NV |
| ADD AH, FFH | AH = 0110 00012 FFH = 1111 11112 + AH = (1)0110 00002 | CY | PL | NZ | NV |
| ADD AX, 2 | AX = 0110 0000 1100 10102 2 = 0000 0000 0000 00102 + 0110 0000 1100 11002 | NC | PL | NZ | NV |

- Fill in the flag mnemonics in the following table.

| Answer |  |  |  |
| --- | --- | --- | --- |
| Set | Flag mnemonics | Clear | Flag mnemonics |
| Overflow | OV | No overflow | NV |
| Direction down | DW | Direction up | UP |
| Interrupt enabled | EI | Interrupt disabled | DI |
| Sign flag negative | NG | Sign flag positive | PL |
| Zero | ZR | Non zero | NZ |
| Auxiliary carry | AC | No auxiliary carry | NA |
| Odd parity | PO | Even parity | PE |
| carry | CY | No carry | NC |

TUTORIAL 4: Computer Architecture

- Given the Figure 1 below.

Figure 1.

- Define Bus in the computer context.     				       (PYP-01/16: 3 marks)

Answer

Bus - Bus is defined as the physical connection. - It is able to transfer data from one location to another in the computer system. - It aims to reduce the number of pathways needed for communication.

- Name the bus configuration as shown in the Figure 1 above.	       (PYP-01/16: 2 marks)

Answer

Multipoint bus.

- List and explain THREE (3) possible types of bus line that can be found in a multipoint bus configuration.							       (PYP-01/16: 6 marks)

Answer

Bus line - Data bus: Carry the data that is being moved from one location to another. - Address bus: Specify the recipient of data on the bus. - Control bus: Provide control for the proper synchronization and operation of the bus and of modules that are connected to the bus.

- Briefly describe TWO (2) differences of point-to-point bus and multipoint bus. Draw a diagram to illustrate each of them.						       (PYP-04/10: 6 marks)

| Answer |  |  |
| --- | --- | --- |
|  | Point-to-point bus | Multipoint bus |
| Definition | A bus carry signals from a specific source to a specific destination | A bus that connect several points together |
| Bus lines | Data bus & control bus | Data bus, address bus & control bus |
| Diagram | Control unit ALU | Computer Computer Computer Computer |

- Memory registers play an important role in the process of Central Processing Unit (CPU) – Random Access Memory (RAM) communication.”

Explain how memory Address Register (MAR) and Memory Data Register (MDR) involved in the CPU-RAM communication.			       		                    (PYP-01/16: 5 marks)

Answer

- MAR is used to hold the memory address that is to be opened for memory data. - MDR is used to hold the data which is being retrieved / to be stored from / to memory location which is currently addressed by MAR. - Both MAR and MDR work together to provide interface for the CPU to communicate with the RAM.

- In the Little Man Computer (LMC) model, Instruction Register (IR), Program Counter (PC), Memory Address Register (MAR), Memory Data Register (MDR) and Accumulator (A) were used.

- Describe the purpose for IR, PC and A respectively.		       (PYP-01/14: 6 marks)

Answer

- IR: Holds the instructions that are currently executed. - PC: Found in CU. Holds the address of the instruction that is going to be executed. - A: Found in ALU. Holds data / result that are used for arithmetic operations / transfer.

- How do IR, PC, A, MAR and MDR relate to each other in the operation of LMC?

(PYP-08/12: 3 marks)

Answer

- PC holds the MAR of the instruction that is going to be executed. - The MDR retrieved indicates the IR that is currently executed. - Based on the address IR, the address for operation is defined. - Load the MDR of the MAR to A. - The current operation is completed and the PC for next instruction is defined.

- Give a scenario or example to support your elaboration in Q4 (b) above.

(PYP-08/12: 4 marks)

Answer

PC MAR MDR IR IR [Address] MAR MDR A PC + 1 PC

- One large modern computer has a 36-bit MAR. How much memory can this computer address?

Answer

Using the formula: 2n where n refers to the size of MAR. 236

- Assuming that LMC model is applied. Suppose that the following instructions are found at the given location in memory:

Program counter: 20

Value in memory location 20: 550 (LOAD)

Value in memory location 21: 151 (ADD)

Value in memory location 22: 350 (STORE)

:

Value in memory location 50: 422

Value in memory location 51: 008

Show the changes of contents in IR, PC, MAR, MDR and A during the execution of Instruction 20, 21 and 22.

| Answer |  |
| --- | --- |
| PC MAR MDR IR IR [Address] MAR MDR A PC + 1 PC | MAR = 20 IR = 550 MAR = 50 A = 422 PC = 21 |
| PC MAR MDR IR IR [Address] MAR A + MDR A PC + 1 PC | MAR = 21 IR = 151 MAR = 51 A = 422 + 008 = 430 PC = 22 |
| PC MAR MDR IR IR [Address] MAR A MDR PC + 1 PC | MAR = 22 IR = 350 MAR = 50 MDR = 430 PC = 23 |

- Show the changes of contents in IR, PC, MAR, MDR and A during the execution of Instruction 20 and Instruction 21.	(PYP-08/15: 8 marks)

Program counter: 20

Value in memory location 20: 550 (LOAD)

Value in memory location 21: 151 (MUL)

Value in memory location 22: 350 (STORE)

:

Value in memory location 50: 2316

Value in memory location 51: 516

| Answer |  |
| --- | --- |
| PC MAR MDR IR IR [Address] MAR MDR A PC + 1 PC | MAR = 20 IR = 550 MAR = 50 A = 2316 PC = 21 |
| PC MAR MDR IR IR [Address] MAR A x MDR A PC + 1 PC | MAR = 21 IR = 151 MAR = 51 A = 2316 x 516 = AF16 PC = 22 |
| If continue at instruction 22: PC MAR MDR IR IR [Address] MAR A MDR PC + 1 PC | MAR = 22 IR = 350 MAR = 50 MDR = AF16 PC = 23 |

- Most modern computers provide a large number of general-purpose registers and very few memory access instructions.

- What is the computer architecture mentioned?			      (PYP-08/13: 2 marks)

Answer

Reduced instruction set computer (RISC )

- Discuss any FOUR (4) advantages of such an architecture.	      (PYP-08/13: 12 marks)

Answer

- Limited and simple instruction set - Execute faster at high clock speed - Do not require complex hardware - Registers-oriented instruction - Reduce memory access - Use registers to operate / hold frequently used instruction - A fixed length / format instruction word - Easy to identify - Can be fetched and decoded independently pipelining - Limited addressing mode - Provide single address mode - Speed up instruction executions - A large bank of registers - Registers are applied widely - Reduce memory access

TUTORIAL 5: Machine Execution

      - Name the FOUR (4) basic components of an assembly language instruction.

Answer

[label: ] operation operand(s) [;comment]

      - What is the rule to follow when the following types of operand is used?

- Memory as operand

Answer

- For 2-operand instructions, memory to memory is not allowed - Example: ADD AH, NUM allow ADD NUM1, NUM2 not allow

- Register as operand

Answer

- Both registers must be the same size - Example: ADD AH, BL

- Immediate value as operand

Answer

- For 2-operand instructions, constants can appear only as the second operand - Example: ADD AH, 40H

      - Issue DEBUG commands for the following operations:

- Reset current IP value to 200.					           (PYP-08/14: 2 marks)

Answer

- R IP IP XXXX :200

- Insert the machine instruction 10 AB 2A 35 and 5D into Code Segment with Offset Address 100.								           (PYP-08/14: 2 marks)

Answer

E CS:100 10 AB 2A 35 5D

- Translate the machine instruction from Offset Address 100 to 108 into assembly code.

(PYP-08/14: 2 marks)

Answer

U 100, 108

- Execute a group of 5 instruction starts from the offset address 100H.     (PYP-08/11: 2 marks)

Answer

P=100,5

- Compare the content at code segment in offset address range 0100H to 010AH with the content in the offset address range 0150H to 015AH		          (PYP-08/11: 2 marks)

Answer

C CS:100 10A 150

      - Explain the following DEBUG operations.

- –D CS:200							          (PYP-01/14: 1 mark)

Answer

Display the content of memory at code segment starts from offset address 0200H

- –A 200

XXXX:0200   SUB   BL,42					          (PYP-01/14: 2 marks)

Answer

Line 1: Assemble (convert symbolic code to object code) at the code segment starts from offset address 0200H. Line 2: At the memory address CS:0200, perform subtraction. BL = BL – 42.

- –T 3								          (PYP-04/14: 1 mark)

Answer

Execute a group of 3 instructions at once.

- H 3064 2130

Answer

Perform Hex computation in addition and subtraction. Result: 5194 (Result of addition) 0F34 (Result of subtraction)

      - Provide the symbolic code for the following operations and trace the execution result for each of them.

- Move the hexadecimal value 4269 to AX register

| Answer |  |
| --- | --- |
| MOV AX,4269 | AX = 4269H |

- Move the 7012H to BX register

| Answer |  |
| --- | --- |
| MOV BX,7012 | BX = 7012H |

- Add the value of BX to AX register

| Answer |  |
| --- | --- |
| ADD AX,BX | AX = AX + BX = 4269H + 7012H = B27BH |

- Exchange the content in AX and BX

| Answer |  |
| --- | --- |
| XCHG AX,BX | AX BX Before exchange: B27BH 7012H After exchange : 7012H B27BH |

      - Describe each of the following DEBUG commands. Complete the table below.

| Answer |  |  |  |
| --- | --- | --- | --- |
| Command | Purpose | Format / E.g. | Output |
| A | Convert symbolic code to object code | -A 100 XXXX:0100 | Object codes. (Not shown.) |
| C | Compare 2 groups of content in memory segment | -C CS:100 105 120 | Segment:offset addresses & content of the selected groups |
| D | Display the content of memory segment | -D CS:100 | Segment:offset addresses, object codes & ASCII codes |
| E | Enter the object to memory segment | -E CS:100 8B | Not shown. Use D command to confirm entered |
| F | Fill in a group of ASCII code repeatedly | -F CS:100 13F “CSA” | Not shown. Use D command to confirm filled in. |
| H | Perform hexadecimal computation | -H 5555 2222 | Result of addition and subtraction |
| P | Proceed to execute a group of instruction | -P=100, 5 | Changes of registers & next instruction |
| Q | Quit the DEBUG program | -Q | Back to root directory |
| R | Retrieve / edit the content of register(s) | -R -R AX AX 0000 :200 | Content of register(s) |
| T | Trace the execution of instruction 1 by 1 | -T -T3 | Changes of registers & next instruction |
| U | Convert object code to symbolic code | U 100, 108 | Segment:offset address, symbolic codes & object codes |

      - Assume that you have used DEBUG program to enter the following E command :

E CS:100 B8 45 01 05 25 00

- Write the command to change the hex value 45 to 54.

Answer

-E CS:101 54

- Write the command to change the instruction pointer address to 200H.

Answer

-R IP IP XXXX :200

      - Assume that the IP register has value 0100H. What is its value after the instruction B8 25 00 is executed?

Answer

IP = 0100H Instruction = B82500H (3 bytes) New IP = 0100h + 3 = 0103H

TUTORIAL 6: Computer Architecture

- Write the command line to perform the followings:

- To assemble a source program named CSA.ASM into listing and object files.

Answer

MASM CSA.ASM

- To link an object file named CSA.OBJ into executable and map files.

Answer

LINK CSA.OBJ

- To directly execute CSA.EXE from DOS.

Answer

CSA ---Or--- CSA.EXE

- To execute CSA.EXE through DEBUG program.

Answer

TD CSA.EXE

- Briefly describe the function of TWO-PASS ASSEMBLER in the process of assembling an Assembly Language program.					            (PYP-08/12: 3 marks)

Answer

Two-pass assembler - Aim: To resolve forward reference to addresses not yet encountered in the program. - Process: - Pass 1: The assembler reads the entire source program and construct a symbol table of names and labels used in the program. The amount of codes to be generated for each instruction is determined during Pass 1. - Pass 2: The assembler uses the symbol table that is constructed in Pass 1 and complete with the object code for each instruction. It then produces, on request, the object (.OBJ), list (LST) and cross reference (.CRF) files.

- Illustrate the process of editing, assembling, linking and executing assembly language programs in an appropriate diagram

Answer

Step 1: Edit (Editor) .ASM Step 2: Assemble (Assembler) .OBJ .LST .CRF Step 3: Link (Linker) .EXE Step 4: Load (Loader) Memory

- Write an assembly program using simplified segment definitions to perform the following operations:

- Move the immediate value hex 40 to the AL register

- Shift AL contents one bit left

- Move immediate value hex 1A to BL

- Multiply AL by BL

| Answer |  |
| --- | --- |
| .MODEL SMALL .STACK 64 .CODE MAIN PROC MOV AL, 40H SHL AL, 1 MOV BL, 1AH MUL BL MOV AX,4C00H INT 21H MAIN ENDP END MAIN | ; (a) AL = 40H ; (b) AL = 40H = 0100 0000B 1000 0000B = 80H ; (c) BL = 1AH ; (d) AX = AL * BL 80H * 1AH = 0D00H |

- Modify your answer in Q4 above for the following requirements:

- Define a 1-byte item named ITEMA containing hex 40 and another named ITEMB containing hex 1A.

- Define a 2-byte item named ITEMC with no constant

- Move the contents of ITEMA to AL and shift left one bit

- Multiply AL by ITEMB

- Move the product in AX to ITEMC

What is the content in the destination register for each of the above instruction? Show your working.

| Answer |  |
| --- | --- |
| .MODEL SMALL .STACK 64 .DATA ITEMA DB 40H ITEMB DB 1AH ITEMC DW 0 .CODE MAIN PROC MOV AX,@DATA MOV DS,AX MOV AL, ITEMA SHL AL, 1 MUL ITEM B MOV ITEMC, AX MOV AX,4C00H INT 21H MAIN ENDP END MAIN | ; (a) ; (a) ; (b) ; (c) AL = ITEM1 = 40H ; (c) AL = 40H = 0100 0000B 1000 0000B = 80H ; (d) AX = AL * ITEMB 80H * 1AH = 0D00H ; (e) ITEMC = AX = 0D00H |

TUTORIAL 7: Assembly Language Fundamentals 1

- Define the following data items in assembly language with the name DATA1 to DATA4 respectively.

(PYP-01/16: 4 marks)

- A 1-byte item containing the hex equivalent to decimal 40.

Answer

DATA1 DB 28H

- A 2-byte item containing an undefined value.

Answer

DATA2 DW 0

- A string with the characters H,E,L,L,O

Answer

DATA3 DB "H","E","L","L","O",”$” DATA3 DB “HELLO$”

- An item containing the consecutive word values 1, 3, 5, 7 and 9.

Answer

DATA4 DW 1,3,5,7,9

- Determine whether each of the following names is VALID or INVALID identifier that conforms to the rules of Assembly Language. Give the reason if it is INVALID.         (PYP-01/09: 10 marks)

- 2NAME

- Total_1

- #TelNo

- .Counter

- @Word

| Answer |  |  |  |
| --- | --- | --- | --- |
| (a) | 2NAME | Invalid | First character cannot be a digit |
| (b) | Total_1 | Valid | - |
| (c) | #TelNo | Valid | - |
| (d) | .Counter | Invalid | First character cannot be a dot “.” |
| (e) | @Word | Invalid | avoid using “@” as the first character because the assembler uses some special words that begin with “@” |

- Below shows a sample of an assembly program. Examine the code. Identify SIX (6) errors found in the program and correct them. You are required to indicate the Line Number in your answer followed by the correct code statements.				            (PYP-04/13: 6 marks)

| Line number | Program content |
| --- | --- |
| Line 1 Line 2 Line 3 Line 4 Line 5 Line 6 Line 7 Line 8 Line 9 Line 10 Line 11 Line 12 Line 13 Line 14 Line 15 Line 16 Line 17 | . MODEL SMALL . STACK 64 . DATA DATA1 DW 1AH DATA2 DB 380 DATA3 DW ? .CODE MAIN PROC FAR MOV AX, DATA MOV DS, AX MOV AX, DATA1 ADD AX, DATA2 MOV DATA3, AX MOV FX, 4C00H INT 21H MAIN END ENDP MAIN |

| Answer |  |  |
| --- | --- | --- |
| Line Number | Error | Correction |
| 5 | DB | DW (DBmax 255) |
| 9 | DATA | @DATA |
| 12 | AX | AL no error, if line 5changed to DW |
| 14 | FX | AX |
| 16 | END | ENDP |
| 17 | ENDP | END |

- Determine the final content of AL register after running the following assembly instructions.

MOV AX, 0H

MOV AL, 34H

MOV BL, 04H

MOV CL, AL

DIV CL

MUL BL

SHL AL, 1

DEC AL						            (PYP-08/15: 2 marks)

| Answer |  |
| --- | --- |
| MOV AX, 0 MOV AL, 34 MOV BL, 04 MOV CL, AL DIV CL MUL BL SHL AL, 1 DEC AL | ; AX = 0000H ; AL = 34H ; BL = 04H ; CL = 34H ; AX = AX / CL AX = 34/34 = 0001H ; AX = AX * BL AX = 0001 *04H = 0004H ; AL = 04H 0000 0100 0000 1000B 08H ; AL = AL – 1 07H |

- Consider the code fragment below. Examine the code and show the contents of the AX and BX registers.								          (PYP-08/13: 10 marks)

.DATA DATA1 DB 32H DATA2 DB 12H .CODE MOV AX, 0004H MOV BX, 0000H MOV BL, DATA1 ; AX= ? , BX = ? SHR BL, 1 ; AX= ? , BX = ? ADD BL, DATA2 ; AX= ? , BX = ? MUL AL ; AX= ? , BX = ? SUB BX, AX ; AX= ? , BX = ?

| Answer |  |
| --- | --- |
| .DATA DATA1 DB 32H DATA2 DB 12H .CODE MOV AX, 0004H MOV BX, 0000H MOV BL, DATA1 SHR BL, 1 ADD BL, DATA2 MUL AL SUB BX, AX | AX = 0004H, BX = 0000H AX = 0004H, BX = 0032H AX = 0004H, BX = 0019H AX = 0004H, BX = 002BH AX = 0010H, BX = 002BH AX = 0010H, BX = 001BH |

- Using only MOV, ADD, SUB, INC, DEC, and NEG, translates the following high-level language assignment statements into assembly language. Assume that A, B, and C are word variables.

- A = B – A

Answer

MOV BX, B SUB BX, A MOV A,BX

- C = A + B

Answer

MOV AX, A ADD AX, B MOV C, AX

- A = - (A + 1)

| Answer 1 | Answer 2 |
| --- | --- |
| MOV AX, A ADD AX, 1 or INC AX NEG AX MOV A , AX | INC A NEG A |

- B = 4 * B + 6

| Answer 1 | Answer 2 | Answer 3 |
| --- | --- | --- |
| MOV AX, B MOV CX, 3 CALC: ADD AX, B LOOP CALC ADD AX, 6 MOV B, AX | MOV AX, 4 MUL B ADD AX, 6 MOV B, AX | MOV BX, B ADD BX,B ADD BX,B ADD BX,B ADD BX,6 MOV B,BX |

- A = B - A – 1

Answer

MOV BX, B SUB BX, A DEC BX or SUB BX, 1 MOV A, BX

- Construct an Assembly Language program using simplified segment directive to calculate the average value based on your inputs. Your program should fulfill the following requirements.

(PYP-08/12: 10 marks)

Marks will be awarded according to the following criteria.

| Requirements | Mark allocation |
| --- | --- |
| Prompt and accept user input of TWO (2) odd integers. Display a comma (,) automatically after the user keys in the first input where the comma is used to serve as a separator between the first and second input | 3 marks |
| Calculate the average using the formula below: (Input1 + Input2) / 2 | 3 marks |
| Calculate and display the average in 1 decimal place | 2 marks |
| Overall program structure | 2 marks |
| Total: | 10 marks |

Sample output :

Enter TWO odd integer values (0 to 9): 5,7 The average of the input values is: 6.0

| .MODEL SMALL .STACK 100 .DATA MSG1 DB "Enter TWO odd integer values (0 to 9): $" MSG2 DB 0DH,0AH, "The average of the input values is: $" INPUT1 DB 0 INPUT2 DB 0 Q DB ? R DB ? TWO DB 2 .CODE MAIN PROC MOV AX,@DATA MOV DS,AX ;---------Display prompt MOV AH,09H LEA DX,MSG1 INT 21H ;---------Get input MOV AH,01H INT 21H SUB AL,30H MOV INPUT1,AL MOV AH,02H MOV DL,"," INT 21H MOV AH,01H INT 21H SUB AL,30H MOV INPUT2,AL | ;--------Output message MOV AH,09H LEA DX,MSG2 INT 21H ;---------Calculate average MOV AL,INPUT1 ADD AL,INPUT2 CBW DIV TWO MOV Q,AL MOV R,AH ;-------Output result MOV AH,02H MOV DL,Q ADD DL,30H INT 21H MOV AH,02H MOV DL,"." INT 21H MOV AH,02H MOV DL,R ADD DL,30H INT 21H MOV AX,4C00H INT 21H MAIN ENDP END MAIN |
| --- | --- |

TUTORIAL 8: Assembly Language Fundamentals II

- Differentiate among the following addresses by completing the table below.

| Answer |  |  |  |
| --- | --- | --- | --- |
|  | Short | Near | Far |
| Distance covered | -128 to 127 (Same segment) | -32768 to 32767 (Same segment) | Another segment |
| Instruction(s) used | JMP Jnnn JE,JNLE LOOP | JMP Jnnn (for 80386/486) | JMP |

- Using int 21h function 01h and LOOP instruction, write a segment of assembly program to accept 3 characters and store them into a variable named INPUT.		          (PYP-08/15: 3 marks)

.DATA

INPUT		DB	 3  DUP (" "), "$"

.CODE

; - Write the assembly codes here –

Answer

.DATA INPUT DB 3 DUP (0) .CODE MOV CX, 3 MOV SI,0 getInput: MOV AH,01H INT 21H MOV INPUT[SI], AL INC SI LOOP getInput

- Based on the assembly program shown below, answer the following questions:

| Line number | Program Contents |
| --- | --- |
| Line 1 Line 2 Line 3 Line 4 Line 5 Line 6 Line 7 Line 8 Line 9 Line 10 Line 11 Line 12 Line 13 Line 14 Line 15 Line 16 Line 17 Line 18 | . MODEL SMALL .STACK 64 .DATA MESSAGE1 DW "HI ASSEMBLY!", 10, 13, "$" ----------------------------------------------------------------------- .CODE MAIN PROC FAR MOV AX, DATA MOV DX, AX MOV CX, 9 L1: MOV AH, 09H LEA DS, MESSAGE1 INT 21H LOOP L1 MOV AX, 4C00H INT 21H MAIN END END MAIN |

- The assembly program consists of FIVE (5) errors. Identify and correct the errors.

(PYP-01/14: 5 marks)

| Answer |  |  |
| --- | --- | --- |
| Line No | Error | Correction |
| 5 | ----- | ;----- |
| 8 | DATA | @DATA |
| 9 | DX | DS |
| 12 | DS | DX |
| 17 | END | ENDP |

- How many times will the message “HI ASSEMBLY!” be displayed on the screen when the program runs?							          (PYP-01/14: 1 mark)

Answer

9 times

- Given the following variable definition in the data segment.

| Data definition | Sample output |
| --- | --- |
| .DATA VAR1 DB 2, 0, 1, 1 | 2011 |

Write a program segment to display consecutively each byte of the data defined in VAR1 using indirect-offset address with LOOP operator.     			         (PYP-08/11: 4 marks)

Answer

.DATA VAR1 DB 2, 0, 1, 1 .CODE MOV CX, 4 MOV SI,0 printOutput: MOV AH, 02H MOV DL, VAR1[SI] ADD DL,30H INT 21H INC SI LOOP printOutput

- Identify and explain how many times each of the following LOOP operations loop:

- MOV CX , 1

L1 :

….

LOOP L1

| Answer |  |
| --- | --- |
| 1 time | Because CX = 1 |

- L2 : MOV CX , 10

…..

LOOP L2

| Answer |  |
| --- | --- |
| Infinitive | Because CX is reset within the loop |

- MOV CX , 10

L3 :

INC CX

LOOP L3

| Answer |  |
| --- | --- |
| Infinitive | Because CX will never reach zero |

- Determine the value in the destination register and the result in the Flags register for OF, ZF, SF, and CF.

- MOV AL , FFH

ADD AL , 1

| Answer |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Instruction | Operation | OF | ZF | SF | CF |
| MOV AL , FFH | AL = FFH = 1111 1111B | NV | NZ | NG | NC |
| ADD AL , 1 | AL = 1111 1111B ADD = 0000 0001B + AL =(1)0000 0000B = 00H | NV | ZR | PL | CY |

- MOV BL , 24H

SUB BL , BL

| Answer |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Instruction | Operation | OF | ZF | SF | CF |
| MOV BL , 24H | BL = 24H = 0010 0100B | NV | NZ | PL | NC |
| SUB BL , BL | BL = 0010 0100B SUB = 0010 0100B - BL = 0000 0000 | NV | ZR | PL | NC |

- MOV CL , 10101010B

ADD CL , 01010101B

| Answer |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Instruction | Operation | OF | ZF | SF | CF |
| MOV CL , 10101010B | CL = 10101010B | NV | NZ | NG | NC |
| ADD CL , 01010101B | CL = 1010 1010B ADD = 0101 0101B + CL = 1111 1111B | NV | NZ | NG | NC |

- MOV DL, 11001100B

ADD DL, 01110011B

| Answer |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Instruction | Operation | OF | ZF | SF | CF |
| MOV DL, 11001100B | DL = 11001100B | NV | NZ | NG | NC |
| ADD DL, 01110011B | DL = 1100 1100B ADD = 0111 0011B + DL =(1)0011 1111B | NV | NZ | PL | CY |

- CHAR_STRING contains uppercase letters that a program is to convert a lowercase. Access each character successively into a register, add 20H to it, and restore it in the string. Use indirect addressing and the LOOP instruction.

(Hint: Use indirect addressing and increment BX for each character)

CHAR_STRING DB "ABCDEFGHIJ"

Answer

LEA BX , CHAR_STRING MOV CX, 10 L10: MOV AL , [BX] ADD AL , 20H MOV [BX] , AL INC BX LOOP L10

8. Given the following table and initializing instructions, show the effect of the MOV

VALUE_TBL DB 1, 2, 3, 4, 5, 6, 7, 8

…

LEA BX, VALUE_TBL

MOV DI, 4

- MOV CL, [BX] ; CL

Answer

1

- MOV DL, [BX + 3] ; DL

Answer

4

- MOV AL, [BX + DI] ; AL

Answer

5

9. 	Using nested loop structure, write an assembly program that print out the right-angled triangle as below:

Sample output :

*** ** *

| Answer |  |
| --- | --- |
| .MODEL SMALL .STACK 100 .DATA .CODE MAIN PROC MOV AX,@DATA MOV DS,AX MOV CX,3 MOV SI,1 ROW: MOV AH,02H MOV DL,0AH INT 21H MOV DL,0DH INT 21H | MOV DI,CX MOV CX,DI COL: MOV AH,02H MOV DL,"*" INT 21H LOOP COL INC SI MOV CX,DI LOOP ROW EXIT: MOV AX,4C00H INT 21H MAIN ENDP END MAIN |

10. Modify the assembly program in Q9 to generate the result as below:

Sample output :

123 45 6

| Answer |  |
| --- | --- |
| .MODEL SMALL .STACK 100 .DATA A DB 31H .CODE MAIN PROC MOV AX,@DATA MOV DS,AX MOV CX,3 MOV SI,1 ROW: MOV AH,02H MOV DL,0AH INT 21H MOV DL,0DH INT 21H | MOV DI,CX MOV CX,DI COL: MOV AH,02H MOV DL,A INT 21H INC A LOOP COL INC SI MOV CX,DI LOOP ROW EXIT:MOV AX,4C00H INT 21H MAIN ENDP END MAIN |

11. Modify the assembly program in Q10 to generate the result as below:

Sample output :

12345 1234 123 12 1

| Answer |  |
| --- | --- |
| .MODEL SMALL .STACK 100 .DATA A DB 31H .CODE MAIN PROC MOV AX,@DATA MOV DS,AX MOV CX,6 MOV SI,1 ROW: MOV AH,02H MOV DL,0AH INT 21H MOV DL,0DH INT 21H | MOV DI,CX MOV CX,DI COL: MOV AH,02H MOV DL,A INT 21H INC A LOOP COL INC SI MOV CX,DI MOV A,31H LOOP ROW EXIT:MOV AX,4C00H INT 21H MAIN ENDP END MAIN |

TUTORIAL 9: Conditional Processing

- Assume that AL and BL contain unsigned data and that CL and DL contain signed data. Determine the CMP and conditional jump instructions for the following.

- Is CL equal to or smaller than DL?

Answer

CMP CL, DL JLE L1

- Is AL equal to or smaller than BL?

Answer

CMP AL, BL JBE L1

- Is AL greater than BL?

Answer

CMP AL, BL JA L1

- Is CL greater than DL?

Answer

CMP CL, DL JG L1

- Does DL contain zero?

Answer

CMP DL, 0 JZ L1

- Is there an overflow?

Answer

CMP CL, DL JO L1

- Assume that DL contains 01111001 and that an item named BOOL_AMT contains 11100011. Determine the effect on DL for the following unrelated operations:

- AND DL , BOOL_AMT

Answer

DL = 0111 1001B BOOL_AMT = 1110 0011B AND DL = 0110 0001B

- OR DL , BOOL_AMT

Answer

DL = 0111 1001B BOOL_AMT = 1110 0011B OR DL = 1111 1011B

- XOR DL , BOOL_AMT

Answer

DL = 0111 1001B BOOL_AMT = 1110 0011B XOR DL = 1001 1010B

- AND DL , 00000000B

Answer

DL = 0111 1001B 0000 0000B AND DL = 0000 0000B

- XOR DL , 11111111B

Answer

DL = 0111 1001B 1111 1111B XOR DL = 1000 0110B

- Write instructions that first clear bit 0 and bit 1 in AL. Then, if the destination operand is equal to zero, the code should jump to label L3. Otherwise, it should jump to label L4.

| Answer | LKN answer |
| --- | --- |
| ; AL = 1010 1011B SHR AL, 1 ; AL = 0101 0101B SHR AL, 1 ; AL = 0010 1010B SHL AL, 1 ; AL = 0101 0100B SHL AL, 1 ; AL = 1010 1000B JZ L3 JNZ L4 | AND AL, 1111 1100B JZ L3 JNZ L4 OR SHR AL,2 SHL AL,2 |

- Translate the following C statements into assembly language.

- if (num1 == num2)

{

X = 1; Y = 2;

}

Answer

MOV BH, num1 CMP BH, num2 JNE L1 MOV X, 1 MOV Y, 2 L1:

- if (al > bl && bl > cl)

{

X = 1;

}

Y = 2;

Z = 4;

| Answer | LKN answer 1 | LKN answer 2 |
| --- | --- | --- |
| CMP AL, BL JBE L1 CMP BL, CL JA L2 L1: … L2: MOV X, 1 | CMP AL, BL JLE L1 CMP BL, CL JLE L1 MOV X, 1 JMP FINISH L1: MOV Y,2 MOV Z,4 | CMP AL,BL JG L1 JMP L3 L1: CMP BL,CL JG L2 JMP L3 L2: MOV X,1 L3: MOV Y,2 MOV Z,4 |

- if (al > bl || bl > cl)

{

X = 1;

}

Answer

CMP AL,BL JA L1 CMP BL, CL JA L1 JMP FINISH L1: MOV X, 1

- Write an assembly program to count the vowels in a character array “This is my favorite”. Print out the number of occurrences of each of the vowels a, e, i, o, and u.

Sample output:

a =1 ;e = 1 ;	i = 3 ; o = 1 ; u = 0

| Answer |  |
| --- | --- |
| .MODEL SMALL .STACK 100 .DATA LIST DB "THIS IS MY FAVORITE$" CA DB 30H CE DB 30H CI DB 30H CO DB 30H CU DB 30H .CODE MAIN PROC MOV AX,@DATA MOV DS,AX MOV SI,0 AGAIN: MOV BH, LIST[SI] CMP BH, "a" JE COUNTA CMP BH, "e" JE COUNTE CMP BH, "i" JE COUNTI CMP BH, "o" JE COUNTO CMP BH, "u" JE COUNTU CMP BH,"$" JE DISPLAY INC SI JMP AGAIN COUNTA: INC CA INC SI JMP AGAIN COUNTE: INC CE INC SI JMP AGAIN COUNTI: INC CI INC SI JMP AGAIN | COUNTO: INC CO INC SI JMP AGAIN COUNTU: INC CU INC SI JMP AGAIN ;--------------DISPLAY DISPLAY: MOV AH,02H MOV DL,0DH INT 21H MOV DL,0AH INT 21H MOV AH,02H MOV DL,CA INT 21H MOV AH,02H MOV DL,CE INT 21H MOV AH,02H MOV DL,CI INT 21H MOV AH,02H MOV DL,CO INT 21H MOV AH,02H MOV DL,CU INT 21H MOV AX,4C00H INT 21H MAIN ENDP END MAIN |

- Using simplified segment directive, write an assembly language program that will find the largest value from a list.							         (PYP-08/13: 15 marks)

The program should:

- Prompt the user to enter 5 decimal digits.

- Accept the user input. (Assume user will only enter 5 digits)

- Find the largest value inside the list.

- Display the largest value on the screen.

Sample output:

Please enter 5 decimal digits >> 35827 The largest value in the list is >> 8

| Answer |  |
| --- | --- |
| .MODEL SMALL .STACK 100 .DATA INPUT DB 5 DUP (0) LARGEST DB 0 .CODE MAIN PROC MOV AX,@DATA MOV DS,AX MOV CX,5 MOV SI,0 GETINPUT: MOV AH,01H INT 21H MOV INPUT[SI],AL INC SI LOOP GETINPUT MOV CL,INPUT[0] MOV LARGEST,CL | MOV SI,1 CHKLARGEST: CMP SI,4 JE DISPLAY MOV BL,INPUT[SI] CMP LARGEST,BL JA MORE JBE LESS MORE: INC SI JMP CHKLARGEST LESS: MOV LARGEST,BL INC SI JMP CHKLARGEST DISPLAY: MOV AH,02H MOV DL, LARGEST INT 21H EXIT: MOV AX,4C00H INT 21H MAIN ENDP END MAIN |

- Refer to the sample output below, you are required to write a complete assembly program to let user to enter a three-character login name and a three-character password for access. If a user enters a correct login name e.g. ABC and a correct password e.g. 123, the access is granted otherwise it is denied.								         (PYP-04/14: 17 marks)

Marks will be awarded based on the following criteria.

| Assessment criteria | Marks allocated |
| --- | --- |
| Declaration of Data Items | 2 marks |
| Initialization of Data Segment | 1 mark |
| Display prompts | 1 mark |
| Receive inputs | 4 marks |
| Validate login name | 3 marks |
| Validate password | 3 marks |
| Display outputs | 2 marks |
| End program | 1 mark |
| Total: | 17 marks |

Sample output:

Login Name: ABC Password: 123 *** Login Successfully! *** Login Name: XYZ Password: 123 *** Invalid Login Name. Access Denied! *** Login Name: ABC Password: 456 *** Invalid Password. Access Denied! ***

| Answer |  |
| --- | --- |
| .MODEL SMALL .STACK 64 .DATA PROMPT1 DB "LOGIN NAME:$" PROMPT2 DB 10, 13, "PASSWORD: $" ERRORMSG1 DB 10, 13, "*** INVALID LOGIN NAME. ACCESS DENIED! ***$" ERRORMSG2 DB 10, 13, "*** INVALID PASSWORD. ACCESS DENIED! ***$" VALIDMSG DB 0DH,0AH,"*** LOGIN SUCCESSFULLY ***$" UNAME DB "ABC" PASSW DB "123" USERN LABEL BYTE MAXN DB 4 ACTN DB ? INPUTN DB 4 DUP ("$") USERP DB 3 DUP (0) .CODE MAIN PROC FAR MOV AX , @DATA MOV DS , AX ; ===== PROMPT AND ACCEPT ; USERNAME AND PASSWORD == MOV AH , 09H LEA DX , PROMPT1 INT 21H MOV AH , 0AH LEA DX , USERN INT 21H MOV AH , 09H LEA DX , PROMPT2 INT 21H MOV CX , 3 MOV BX , 0 L1: MOV AH , 01H INT 21H MOV USERP[BX],AL INC BX LOOP L1 | ; ===== AUTHENTICATE THE ; USERNAME AND PASSWORD === MOV CX , 3 MOV SI , 0 L2: MOV AL , INPUTN[SI] CMP AL, UNAME[SI] JNE ERROR1 INC SI LOOP L2 MOV CX , 3 MOV DI , 0 L3: MOV AL , USERP[DI] CMP AL, PASSW[DI] JNE ERROR2 INC DI LOOP L3 JMP VALID ; ===== AUTHENTICATION RESULT MESSAGE ===== ERROR1: MOV AH , 09H LEA DX , ERRORMSG1 INT 21H JMP EXIT ERROR2: MOV AH , 09H LEA DX , ERRORMSG2 INT 21H JMP EXIT VALID: MOV AH , 09H LEA DX , VALIDMSG INT 21H EXIT: MOV AX, 4C00H INT 21H MAIN ENDP END MAIN |

TUTORIAL 10: Keyboard and Screen Processing

            - List out the 3 main usages of the INT 21H operations.

Answer

- Input - Output - Terminate processing

            - What are the purposes of the following functions?

- INT 10H function 02H

Answer

Set cursor

- INT 10H function 06H

Answer

Scroll screen

- INT 21H function 01H

Answer

Input byte

- INT 21H function 02H

Answer

Output byte

- INT 21H function 07H

Answer

Input byte (no echo)

- INT 21H function 09H

Answer

Output string

- INT 21H function 0AH

Answer

Input string

            - Write the instructions required to set the cursor to row 12, column 24. Then rewrite the instructions set so that it involves DX register.  go to question 11

Answer

MOV AH, 02H ; set cursor location MOV BH, 00 ; page number MOV DH, 11 or 0BH ;ROW MOV DL, 23 or 17H ;COL INT 10H ---Using DX register--- MOV AH, 02H MOV BH, 00 MOV DX, 0B17H INT 10H

            - Explain the effect of the following program segment?

MOV AX , 0600H

MOV BH , 71H

MOV CX , 0000H

MOV DX , 184FH

INT 10H

| Answer |  |
| --- | --- |
| MOV AX , 0600H MOV BH , 71H MOV CX , 0000H MOV DX , 184FH INT 10H | ; Clear screen and set screen scrolling ; 1st for background color, 2nd digit for foreground color ; start coordinate ; end coordinate ; Call interrupt service for screen monitoring |

            - “When using INT 21H function 09H for displaying, a delimiter ($) is defined immediately following the display area.”

- Why the delimiter is needed?

Answer

To mark the end of a string

- What will happen if the delimiter is missing?

Answer

The remaining data will be retrieved.

            - INT 21H function 0AH for keyboard input requires a parameter list.

- What will the first byte store?

Answer

Maximum length of the parameter

- What will the second byte store?

Answer

Actual length of the parameter

- What will the continuous bytes store?

Answer

Actual parameter data

            - What register works closely with INT 21H to perform various functions?

Answer

AH register

            - In order to display a character using INT 21H function 02H, the character to be displayed must be loaded to which register?

Answer

DL register

            - Write instructions to reset cursor in the new line and at the left most position.

Answer

MOV AH, 02H MOV DL, 0AH INT 21H MOV DL, 0DH INT 21H

            - For INT 21H function 09H, when LEA command is applied, where will the assembler load the effective (offset) address to?

Answer

DX register

            - Identify the hex values of AH, DX and INT if the cursor is set at row 7, column 14 on screen using assembly language.						        (PYP-04/14: 3 marks)

Answer

MOV AH, 02H MOV BH, 00 MOV DX, 060EH INT 10H

            - Using simplified segment directive, write an assembly language program that performs One’s Complement operation. 						         (PYP-08/14: 15 marks)

The program should:

- Prompt user to enter an 8-bit binary data.

- Accept the 8-bit binary data from the user. (Assume user will only enter 8-bit binary data)

- Flip the binary data accepted.

- Display the One’s Complement result on the screen.

Marks will be awarded based on the following criteria.

| Assessment criteria | Marks allocated |
| --- | --- |
| Print prompts | 3 marks |
| Accept input (with proper data structure) | 2 marks |
| Flip operation using loop instruction | 8 marks |
| Print result | 2 marks |
| Total: | 15 marks |

Sample output:

Please enter 8-bit binary data >> 10100101 The One’s Complement value of the data is >> 01011010

| Answer |  |
| --- | --- |
| .MODEL SMALL .STACK 100 .DATA INPUT DB 8 DUP (0) STR1 DB "PLEASE ENTER 8-BIT BINARY DATA >> $" STR2 DB 0DH,0AH,"THE ONE’S COMPLEMENT VALUE OF THE DATA IS >> $" .CODE MAIN PROC MOV AX,@DATA MOV DS,AX MOV AH,09H LEA DX,STR1 INT 21H MOV CX,8 MOV SI,0 GETINPUT: MOV AH,01H INT 21H MOV INPUT[SI], AL INC SI LOOP GETINPUT MOV SI,0 CHK: CMP SI,7 JE DISPLAY CMP INPUT[SI],"0" JE CHG1 JNE CHG0 | CHG1: MOV INPUT[SI],"1" INC SI JMP CHK CHG0: MOV INPUT[SI],"0" INC SI JMP CHK DISPLAY: MOV AH,09H LEA DX,STR2 INT 21H MOV CX,8 MOV SI,0 PRINTINPUT: MOV AH,02H MOV DL,INPUT[SI] INT 21H INC SI LOOP PRINTINPUT EXIT:MOV AX,4C00H INT 21H MAIN ENDP END MAIN |

            - Write an assembly program that will prompt the user for input. The program will display all the numeric data input by the user, while filters all other characters. (Assume user will not enter $.)

Sample output:

Please enter any data and press enter >> 12A3%C&4 The numerical value is >> 1234

| Answer |  |
| --- | --- |
| .MODEL SMALL .STACK 64 .DATA S1 DB "PLEASE ENTER ANY DATA AND PRESS ENTER>> $" S2 DB 0DH,0AH,"THE NUMERICAL VALUE IS >> $" ARRAY LABEL BYTE MAX DB 100 ACT DB 0 ARRAYDATA DB 100 DUP ("$") .CODE MAIN PROC MOV AX,@DATA MOV DS,AX ;----OUTPUT STR MOV AH,09H LEA DX,S1 INT 21H ;--INPUT STR MOV AH,0AH LEA DX,ARRAY INT 21H ;----OUTPUT STR2 MOV AH,09H LEA DX,S2 INT 21H | ;---CHECKNUMBER MOV SI,0 CHKNUM: CMP ARRAYDATA[SI],"$" JE EXIT CMP ARRAYDATA[SI],"0" JAE CHK2 JB CHKNEXT CHK2: CMP ARRAYDATA[SI],"9" JBE PRINTNUM JA CHKNEXT CHKNEXT: INC SI JMP CHKNUM PRINTNUM: MOV AH,02 MOV DL,ARRAYDATA[SI] INT 21H INC SI JMP CHKNUM EXIT: MOV AX,4C00H INT 21H MAIN ENDP END MAIN |

TUTORIAL 11: Input and Output Facilities

- Define I/O module. Discuss the roles of I/O module in CPU – I/O communication.

(PYP-01/14: 2, 8 marks)

Answer

I/O module - It serves as an interface between the CPU and the specific device. - It accepts commands from the CPU on one side and control the device on the other. - Roles: - Recognizes messages addressed to it and accepts commands from the CPU - Provides a buffer where the data from memory can be held until it can be transferred to the disk. - Provides the necessary registers and controls to perform a direct memory transfer. - Provides interrupt capability to notify the CPU when the operation is completed.

- Differentiate the following I/O techniques: Programmed I/O and Direct Memory Access (DMA) in term of their characteristics.					          (PYP-01/14: 10 marks)

| Answer |  |  |
| --- | --- | --- |
|  | P I/O | DMA |
| Devices involved | Slow devices | Fast devices |
| Data size involved | Single I/O | Block of data |
| Machine cycle | Full | Initiation & notification |
| CPU involvement | Full | Partial |
| Data required | - Address - Data | - Location of data in device - Location of data in memory - Data size - Direction of transfer |
| Requirement | Direct connection | - I/O module & memory are connected - I/O modules are capable to read / write - Conflict between I/O module & CPU must be avoided |

- An I/O technique is known as Interrupt-driven I/O. Explain the technique.    (PYP-04/14: 5 marks)

Answer

Interrupt-driven I/O - Signals that causes the CPU to alter its normal flow on instruction execution. - Advantages: - - Free CPU from waiting for events - Provides control for external input - Examples: - - An unexpected user input - Illegal instructions - Multitasking, multiprocessing - An abnormal situation (paper jam, out of paper, bad sector)

- Explain the FOUR (4) usages of Interrupt.			        	           (PYP-08/14: 8 marks)

Answer

- As an external event notifier - As a completion signal - As a mean of allocation of CPU time - As abnormal event indicator

- How does Direct Memory Access (DMA) release the waiting time of CPU?  (PYP-04/14: 3 marks)

Answer

- The transfer is initiated by a program in the CPU by using PIO, but the CPU can then bypass for the remainder of the transfer. - The I/O module will notify the CPU with an interrupt when the transfer is completed. - Once this has occurred, the data is in memory, ready for the program to use.

- Interrupt is a signal sent to the CPU to indicate an event that requires immediate attention. Servicing the interrupt is a process where the CPU branches to the interrupt handler program from its current program. Explain the flow of servicing an interrupt in detail.        (PYP-08/13: 4 marks)

Answer

- Suspends program in progress - Save context, including last instruction executed and data values in registers, in PCB (process control block). - Branches to interrupt handler program (interrupt routine) - Resume control to the interrupted program by referring to the PCB.

- Suppose you are sending a block of data from a disk to memory.

- Identify the most appropriate I/O handling technique to support the data transfer process as mentioned. Explain your choice.					           (PYP-08/13: 2 marks)

Answer

Direct memory access (DMA)

- Based on the scenario given in Q7, explain the process of the I/O handling technique that you mentioned in Q7 (a) with the aid of an appropriate diagram.	           (PYP-04/14: 6 marks)

Answer

- PIO is used to prepare I/O module for transfer by providing required info & initiating transfer. - DMA transfer. In this case, data is transferred from disk to memory. - Upon completion, disk controllers sends completion interrupt to CPU. CPU; I/O module (disk controller; Disk. Memory.

- Distinguish between a bus architecture and a channel architecture in terms of their characteristics and operations.							          (PYP-08/12: 6 marks)

| Answer |  |  |
| --- | --- | --- |
| I/o architecture | Bus architecture | Channel architecture – for IBM mainframe computer |
| Nature | Simple | Complex |
| Application | PC | Mainframe |
| Components | Backbone + interface | Channel subsystem (I/O processor) |
| Pros | Easy expand & flexible access | Better control & free CPU |
| Cons | Loose management | Max 8 channel path |
| Diagram |  |  |

- Four pieces of data must be provided to the I/O controller for a particular I/O device to initiate a Direct Memory Access (DMA) transfer. What are they?		          (PYP-04/10: 4 marks)

Answer

- The location of data on the I/O device - The starting location of the block of data in memory - The size of block to be transferred - The direction of transfer. Read (I/O memory) write (memory I/O)

- In the event of multiple interrupts, illustrate how these multiple interrupts could be handled by using vectored interrupt and polled interrupt respectively. Support your answer with a diagram for each.

| Answer |  |
| --- | --- |
| Vectored interrupt | Polled interrupt |
| The address of the interrupting device is included as part of the interrupt. | The general interrupt that shared by all devices. Identifies the interrupting device by polling each device. (check device one by one) |

(PYP-01/12: 10 marks)

- Consider the interrupt that occurs at the completion of a disk transfer (Disk to Memory).

- “Who” is interrupting “Whom”?

Answer

Disk is interrupting CPU.

- Why is the interrupt used in this case? What would be happened if there were no interrupt capability on this computer?

Answer

To notify the completion of transfer. The resources cannot be released to the others.

TUTORIAL 12: Computer Architecture

- Briefly explain how a system performance is affected.

Answer

Factors affecting the performance of a system are including the following: - Multiple CPUs - Faster system clock speed - Wider instruction and data path - Faster disk access - Memory access time - Increase amounts of memory and faster memory Briefly explain for each of them.

- What is the main different between loosely-coupled system and parallel system?

(PYP-01/10: 6 marks)

Answer

- Distributed Operating systems are also referred to as Loosely Coupled systems whereas parallel processing systems are referred to as tightly coupled systems. - A Loosely coupled system is one in which the processors do not share memory and each processor has its own local memory whereas in a tightly coupled system there is a single system wide primary memory shared by all the processors. - The processors of distributed operating systems can be placed far away from each other to cover a wider geographic area which is not the case with parallel processing systems. - The no. of processors that can be usefully deployed is very small in a parallel processing operating system whereas for a distributed operating system a larger no. of processors can be usefully deployed. - In the distributed operating system there is an unpredictable communication delays between processors whereas the processors in the parallel processing system share over an interconnection network

- Cache memory is used as a technique to enhance memory performance.

- With the aid of a diagram, explain how a cache memory helps to enhance memory performance.

(PYP-08/13: 5 marks)

Answer

Cache memory - Definition: a small amount of high speed memory between CPU & main storage, which is invisible to programmer and cannot be directly addressed by CPU. - Diagram: CPU Memory Cache memory

- What will happen when there is a cache miss?                                          (PYP-08/13: 2 marks)

Answer

- If there is a miss, a miss requires the cache controller to select a line for replacement from memory.

- Analyze the impact of cache miss on the execution of instruction.           (PYP-08/13: 4 marks)

Answer

- Data not found in cache. Processor loads data from Memory and copies into cache. This results in extra delay.

- Explain what will happen when a cache memory is full.	           (PYP-08/15: 4 marks)

Answer

- When cache memory is full, some block in cache memory must be selected for replacement, usually based on a least recently used basis.

- List and explain TWO (2) configurations of a multiprocessing system.           (PYP-08/13: 6 marks)

Answer

Two ways of configuring a multiprocessing system are: - Tightly coupled systems - Also called multiprocessor systems - Characteristics: identical access to programs, data, shared memory, I/O, etc. - Loosely coupled systems - Also called cluster / multi-computer systems - Characteristics: each system has its own CPU, memory and I/O facilities.

- Explain the following techniques in enhancing memory performance.

- Wider path for memory access

Answer

- Retrieve multiple bytes instead of 1 byte at a time - Several bytes / words can be read / written between CPU and memory with each access.

- Cache memory							           (PYP-04/13: 4 marks)

Answer

- A small amount of high speed memory between CPU and main memory storage - It is invisible to the programmer and cannot be directly addressed in the usual way.

- What performance improvement is offered by the use of memory interleaving?

Answer

Performance improvement offered by memory interleaving: - Increase the effective rate of memory access by dividing memory into parts - Allow memory access to more than one location at a time.

- Explain how pipelining serves to reduce the average number of steps in the execution part of the fetch-execute cycle.

Answer

The pipelining - Definition : It overlaps instruction so that more than one instruction is being worked on at a time. - Process : Each instruction completes a step, the following instruction moved into the stage jus vacated. Thus, when the first instruction is completed, the next instruction is already one stage short of completion. Fetch; Decode; Execute; Store. Fetch; Decode; Execute; Store. Fetch; Decode; Execute; Store. Instruction3 Instruction2 Instruction1 Time 0 1 2 3 4 5

- The designer of a new operating system has proposed the use of virtual storage memory management for real time processing so that the system can handle programs that are too large to fit in the limited memory space. What are the implications of this decision in term of the way that virtual storage works?

Answer

Virtual storage - Only the active pages require corresponding frames in the page table and in physical memory. - Load only a small part of a program and have it executed. - Process: - In the case the two requirements met, the instruction will be executed. - The instruction / data must be in physical memory. - The page table for the program must contain an entry that maps the virtual address being accessed to the physical location containing the instruction / data. - In the case of not enough frames available to populate the page table when the program is loaded, load only a small part of a program and have it execute.

- Differentiate between logical address and physical address.

Answer

- Physical addresses do not need to be consecutive. - Logical addresses mapped to physical addresses.

- Complete the diagram below.

Answer

(a) 130 (b) 100 (c) 450